5G LAB-IN-A-BOX

Live Demo Runbook

GTP/PFCP anomaly detection on the NVIDIA DGX Spark (GB10)

Story arc: “This is a real 5G network → here’s an attack → the GPU model catches it.” Budget about 15 minutes of prep and a 15-minute live demo. All commands run on the DGX, from ~/Projects/5g-lab-in-a-box.

| Beat | Time | What you show |
| --- | --- | --- |
| Prep | ~15 min | Bring the lab up, verify egress, pre-capture the dataset, open the deck |
| 1 · Frame | 2 min | Deck slides 1–3: what we detect and why anomaly detection |
| 2 · Real 5G | 3 min | The live core, UE attached, a phone reaching the internet through the tunnel |
| 3 · Attack | 3 min | Fire a GTP-U flood at the UPF from a container |
| 4 · Detection | 3 min | Run the detector on the GPU — ROC-AUC 1.000, 100% recall |
| 5 · How | 4 min | Deck slides 7–13: autoencoder, reconstruction error, results, interpretation |

# 1 · Before peers arrive  (~15 min)

Bring the lab up and verify the data path

The lab may have stopped over your break. Start the core (skipping the amd64-only WebUI), start the RAN, and confirm the UE can reach the internet:

cd ~/Projects/5g-lab-in-a-box
# core — 12 NFs
docker compose -f deploy/open5gs/docker-compose.yml --env-file deploy/open5gs/.env up -d \
  mongo nrf scp amf ausf udm udr pcf bsf nssf smf upf
docker compose -f deploy/open5gs/docker-compose.yml ps        # all "Up"
# RAN — gNB + UE
CORE_NET=open5gs-core deploy/ran/ueransim/run-containers.sh up
docker exec ue ping -c3 -I uesimtun0 8.8.8.8                  # must reply

The test subscriber persists in the Mongo volume, so you should not need to re-provision. If the ping fails, the UE likely dropped — re-run the run-containers.sh up line.

Pre-capture the dataset (do this now, not live)

The full labeled capture takes ~7 minutes — dead air in front of an audience. Run it during prep so the features are ready, then you can train live in seconds:

sudo -v                                   # cache sudo, avoids mid-run prompts
capture/run_labeled_dataset_docker.sh
python3 capture/extract_features.py capture/pcaps/*.pcap \
  -o capture/data/features.parquet --window 1.0

Finally: open the deck (docs/decks/5g-gpu-detection.pptx) and run the detector once so it is warm.

# 2 · The live demo  (~15 min)

Beat 1 — Frame it  (deck, 2 min)

Open on slides 1–3: what you are detecting (N3/N4 user-plane attacks) and the core idea — learn what “normal” traffic looks like, then flag anything unfamiliar.

Beat 2 — “This is a real 5G network”  (3 min)

Show the running core, the UE attach, and a device reaching the internet through the 5G tunnel:

docker compose -f deploy/open5gs/docker-compose.yml ps     # the live core
docker logs ue --tail 6                                    # "PDU Session ... successful"
docker exec ue ping -c3 -I uesimtun0 8.8.8.8               # a phone on the internet

Beat 3 — “Here comes an attack”  (3 min)

Fire a GTP-U flood at the UPF from a throwaway container — punchy and visual:

UPF_IP=$(docker run --rm --network open5gs-core python:3.11-slim \
  getent hosts upf | awk '{print $1}')
docker run --rm --network open5gs-core -v "$PWD/attacks":/attacks:ro python:3.11-slim \
  python /attacks/gtpu/gtpu_flood.py --target "$UPF_IP" --i-own-this-lab --count 5000 --rate 400

Beat 4 — “The GPU model catches it”  (3 min)

Run both detectors on the dataset you pre-captured:

python3 detector/baseline.py --data capture/data/features.parquet --model both

Point at the output live: [autoencoder] training on GPU: NVIDIA GB10, ROC-AUC 1.000, and recall 1.000 on all four attack classes.

Beat 5 — “How it works”  (deck, 4 min)

Close on deck slides 7–13: the autoencoder, reconstruction error as the anomaly score, the results table, and the honest interpretation.

# 3 · Keep it smooth

- If the UE dropped or the UPF was restarted, the ping fails — re-run CORE_NET=open5gs-core deploy/ran/ueransim/run-containers.sh up to re-attach.

- Keep the deck open as a full fallback — if anything live misbehaves, the slides carry the whole story on their own.

- Lean into the honesty on slide 13: the perfect score proves the pipeline works end-to-end, not production accuracy. Technical peers respect that framing — and it heads off the obvious “1.000 is too good” question.

- The subscriber and captured data persist between runs, so a mid-demo restart of the core recovers quickly.

5G Lab-in-a-Box  ·  Live demo runbook  ·  GPU anomaly detection on the NVIDIA DGX Spark