5G LAB-IN-A-BOX  ·  PHASE 2A

Network Slicing in 5G — Speaker Notes

Companion notes for the deck “5g-network-slicing.pptx”. One block per slide — what to say, in plain language. Aim ~1 minute per slide (~10-minute talk).

Slide 1  ·  Title — Network Slicing in 5G

Open with the promise: “5G lets one physical network act as many separate networks at once — that’s network slicing. Today I’ll cover three things: what slicing actually is, what we built in our lab, and how we proved it really works — not in theory, but with a running system where I can show you the traffic staying separated.”

Tip: Point at the three badges — two slices, two user planes, isolation proven — so the audience knows this ends in a live-proof, not just concepts.

Slide 2  ·  What is network slicing?

Define it simply: “A slice is a logical network carved out of one physical 5G network. All the slices share the same towers, the same hardware — but each behaves like its own private network, tuned for a purpose and isolated from the others.” Then introduce the name: “Each slice has an identifier called an S-NSSAI — think of it as the slice’s label — made of a slice type (SST) and an optional differentiator (SD).”

Use the highway analogy on the right: “One highway, several lanes — an express lane for fast traffic, a freight lane for bulk, an emergency lane kept clear. Same road, separated lanes. That’s exactly what slicing does to a mobile network.”

Slide 3  ·  Why it matters

Explain the motivation: different services have opposite needs. “Streaming video wants raw speed. A remote-control or safety application wants ultra-low latency and rock-solid reliability. A field of IoT sensors wants to connect millions of cheap devices. One-size-fits-all can’t do all three well.” Walk the three cards — eMBB, URLLC, and massive IoT.

Land the payoff — isolation: “The real win is isolation. Each slice gets its own resources, so a traffic surge, a failure, or even an attack in one slice doesn’t spill into another. For an operator that’s two things at once: reliability, and a way to sell differentiated, guaranteed services.”

Slide 4  ·  How it works — shared brain, separate lanes

Give the one-sentence mental model: “The clever part of slicing is what’s shared versus split. The control plane — the signalling brains, the AMF, SMF, and NSSF — plus the radio, are shared across all slices. But the user plane, the part that actually carries your data, is split: each slice gets its own UPF, its own data network name, and its own IP address range.”

Name the two functions that do the work: “Two functions make it happen. The NSSF — the Network Slice Selection Function — decides which slice a device is allowed to use. Then the SMF steers that device’s session to the correct UPF, matching on the data-network name. Right slice in, right user plane out.”

Slide 5  ·  What we built — two slices, two user planes

Walk the diagram left to right: “Here’s our lab. On the left, two devices — UE-A on an eMBB slice, UE-B on an IoT slice. In the middle, a shared gNB and a shared control plane. On the right, the split: slice A’s data goes to one UPF on the internet network, 10.45; slice B’s goes to a completely separate UPF, upf-iot, on the iot network, 10.46.”

Reinforce the key line at the bottom of the diagram: “Control plane and radio shared; user plane split per slice — each device’s data rides only its own UPF.” Use the side panel to state the two slices’ exact parameters.

Tip: This is the anchor slide — if someone only remembers one picture, it should be this one.

Slide 6  ·  Under the hood — what we configured

Keep this brisk — it shows depth without drowning people: “Turning our single-slice lab into two slices took six changes. We told the AMF to advertise a second slice. We mapped both slices in the NSSF. We gave the SMF two data-network-tagged UPFs and had it advertise both slices. We stood up a second UPF — upf-iot — on its own subnet. We provisioned a subscriber allowed on slice two. And on the radio side, the gNB now advertises both slices and we added a second device.”

Close with the reassurance: “Every change was additive — the original single-slice setup keeps working exactly as before.”

Slide 7  ·  How a device lands on the right slice

Narrate the flow as a story: “When a device connects, it asks for a slice by its S-NSSAI. The AMF checks with the NSSF — and with the device’s subscription — whether it’s allowed on that slice. If yes, an SMF is chosen for that slice, it routes to the matching UPF by data-network name, and the UPF hands out an IP from its own pool.”

Emphasize the gate: “Note the subscriber data at the bottom — a device can’t just pick any slice. What it’s allowed to use is stored in its subscription. That’s the control that stops a device from wandering into a slice it shouldn’t.”

Slide 8  ·  The demo — two UEs, two subnets

Present the result plainly: “Here’s the demo. Both devices attached to the same core, through the same gNB. But look at the addresses — UE-A got 10.45.0.2, UE-B got 10.46.0.2. Different pools, from two different UPFs. And both reached the internet successfully through their own UPF.”

Set up the next slide: “Different addresses is the first visible sign the two slices are real. But different labels alone don’t prove separation — so let’s actually watch the traffic.”

Slide 9  ·  The proof — isolation

This is the money slide — deliver it with confidence: “We captured the traffic on each UPF’s data interface while both devices were pinging. On the first UPF we saw only 10.45 traffic — slice A’s device. On the second UPF, only 10.46 — slice B’s. Neither UPF ever saw the other slice’s traffic.”

State the conclusion: “That’s not a label or a policy — that’s physical separation of the data paths. The two slices genuinely ride separate user planes. Isolation, proven.”

Tip: If it’s a live audience, this is where you can run the capture on the box and show it in real time — it’s only a few seconds.

Slide 10  ·  So what — slicing as a containment boundary

Zoom out to the significance: “We built and proved real network slicing — two slices, two user planes, isolation shown by capture. And here’s the part that matters beyond performance: isolation is a security property. A surge, a failure, or an attack in one slice stays contained.”

Close on the roadmap: “That makes a slice a containment boundary — the foundation for our next steps: an edge/MEC demo that reuses this same plumbing, and a security demo where we attack one slice and show the others stay clean, with our anomaly detector scoped to each UPF.” Thank the audience and invite questions.